NBS SPECIFICATION

K32 Panel Cubicles / Duct & Wall Linings / Screens

Ascent Range Vanity Unit System

| PROJECT [Insert project name] | PREPARED FOR [Architect / Contractor] | DATE October 2026 | REVISION / STATUS A — For Approval |
| --- | --- | --- | --- |

130   VANITY UNITS — PANEL SYSTEMS

| Location | Refer to Architect's drawings. |
| --- | --- |
| Manufacturer | Bridgewater Direct, Unit 1C Britannia Business Park, Union Road, Bolton BL2 2HP. Tel: 0161 703 3980 · sales@bridgewaterdirect.co.uk · www.bridgewaterdirect.co.uk |
| Product reference | Ascent Range Vanity Units — MR MFC vanity unit system for damp and humid washrooms |
| Panels | 18 mm moisture-resistant melamine faced chipboard (MR MFC), decorative finish both faces, to BS EN 14322. |
| Panel thickness | 18 mm |
| Edge treatment | Square edge, with matching 2 mm ABS edging to all exposed edges. |
| Worktop height | 850 mm adult, 650 mm junior or 550 mm infant, floor to worktop surface. |
| Depth | 600 mm (fully recessed basins) or 400 mm (semi-recessed). Made to measure, 250–700 mm. |
| Width & layout | 1–6 basins per run, made to measure. Between walls, corner, freestanding or L‑shaped. |
| Upstand & downstand | 100 mm upstand and 200 mm downstand, factory fitted. |
| Access panels | Removable front access panels, supplied loose to fit after the plumbing is tested. |
| End panels | Exposed ends in decorative finish; ends against walls in service finish. |
| Alternative top | White solid surface trough with a tap hole and plug to each basin, optional 100 mm plant-on upstand. For units 600 mm deep or over. |
| Panel colour / finish | To be confirmed by Architect. Full decor range and samples available on request. |

250   INSTALLATION

- Programming: Do not install vanity units until the building is weathertight, wet trades have completed their work, wall and floor finishes are complete and the building is dried out.

- Modifications: Do not cut, plane or sand prefinished components. Basin and tap openings are cut on site: seal all cut edges. Basins, taps and wastes by others.

- Fixing: Secure units to a solid background or studwork using fasteners and methods recommended by the manufacturer, to prevent bowing or other distortion of carcass, worktop and panels.

- Moisture and movement: Make adequate allowance for future movement.

SAMPLES & TECHNICAL SUPPORT

Decor samples, datasheets and project advice are available direct from Bridgewater Direct: call 0161 703 3980 or email sales@bridgewaterdirect.co.uk quoting the product reference above, or configure online at www.bridgewaterdirect.co.uk.