NBS SPECIFICATION

K32 Panel Cubicles / MFC

Smart Range Toilet Cubicle System

| PROJECT [Insert project name] | PREPARED FOR [Architect / Contractor] | DATE October 2026 | REVISION / STATUS A — For Approval |
| --- | --- | --- | --- |

112   CUBICLE SYSTEM

| Location | Refer to Architect's drawings. |
| --- | --- |
| Manufacturer | Bridgewater Direct, Unit 1C Britannia Business Park, Union Road, Bolton BL2 2HP. Tel: 0161 703 3980 · sales@bridgewaterdirect.co.uk · www.bridgewaterdirect.co.uk |
| Product reference | Smart Range Toilet Cubicles — MFC cubicle system for dry washrooms |
| Panels | 18 mm melamine faced chipboard (MFC), decorative finish both faces, to BS EN 14322. |
| Panel thickness | 18 mm doors, pilasters and partitions. |
| Edge treatment | Square edge, with matching 2 mm ABS edging to all exposed edges. |
| Overall height | 1950 mm, with 150 mm floor clearance (1800 mm panels). |
| Partition depth | 1500 mm standard. Made to measure, 1400–1800 mm. |
| Width & layout | 800–1200 mm per cubicle, 1–6 per run. Between walls, left or right corner, or freestanding. |
| Doors | Overlap the pilasters by 20 mm each side. Inward or outward opening, left or right hand hung. Outward opening doors 50 mm shorter to clear the headrail. |
| Supports | Adjustable cylindrical legs: one per partition, two per middle pilaster 300 mm wide or over. |
| Headrail | D-shaped aluminium headrail with end caps. Top aluminium channel available instead. |
| Fittings & ironmongery | Satin anodised aluminium (SAA) throughout: heavy-duty hinges, indicator latch and bolt, channel brackets and coat hook. |
| Panel colour / finish | To be confirmed by Architect. Full decor range and samples available on request. |

250   INSTALLATION

- Programming: Do not install cubicles or duct wall panels until the building is weathertight, wet trades have completed their work, wall and floor finishes are complete and the building is dried out.

- Modifications: Do not cut, plane or sand prefinished components.

- Fixing: Secure components using fasteners and methods recommended by the cubicle manufacturer, to prevent bowing or other distortion of frames, panels and doors.

- Moisture and movement: Make adequate allowance for future movement.

SAMPLES & TECHNICAL SUPPORT

Decor samples, datasheets and project advice are available direct from Bridgewater Direct: call 0161 703 3980 or email sales@bridgewaterdirect.co.uk quoting the product reference above, or configure online at www.bridgewaterdirect.co.uk.